- Influence factors of distortion:

Model of saturation (amp) and phase – python script

Python script models: window functions – modulated to IF, up conversion to RF, then non-linearities, power detector at o/p (side lobes in freq), back into Rx, down conversion to baseband, pulse compression filters at end (side lobes in time – ghost targets seen) – classes for diff blocks

Draw them first

O/p signal (Tx with distortion), receiver, pulse compression

1st investigations

- Model Tx chain (with meas. Equip.)

Rx chain considered linear

- Available sensors:

Power detector at o/p for transmitted pulses (only amp)

Estimate phase shift

Power reflections, so could see Rx chain maybe

- Predistortion feedback loop

Simulation: May need to upsample for PA non linearities (To check: after applying the PA, plot the output spectrum and see whether the regrowth skirts have decayed into the noise floor before they reach ±fs/2)

Sensor for this

Open loop – i/p changes depending on what we measure at o/p

Closed loop – estimate this

Amp and phase loops

Going from model to getting the inverse function

“Table of contents – structuring the work

Understanding predistortion – book on overview

Model the stuff simply – to understand system.”

Settings.xml parametres for window function, IF frequencies, RF frequencies

Available models for predistortion. Simple ones first – AM,AM conversion

Second model with AM PM

Third model with memory effects

Find appropriate model for TX chain and doing the modeling (param fitting)

Git repository – start local and move it later

S-Band Class C

X-Band Class A and B

13 April 2026

Draft up piece of paper of full chain for your understanding

Need Filter after RF down conversion to remove image

Look at other filters (and components) in FEC Commissioning Doc and see if you need to incorporate them too (e.g. harmonic filter, IF complex filter etc)

8 May 2026

- Set Saleh Model to match linear part of curve, set alpha to 1 – really understand what model is doing

- MODEL Undistorted vs distorted vs predistorted then can set parametres to tune and determine KPIs like side lobe suppression level, SNR, amplitude etc…

- Model predistortion

- Can we improve the side lobe suppression?

- Can we improve the pulse compression magnitude?